INDIGO Electronics - DIY Home Automation

KAT5 AV Link Transceivers AV Multi-zone Switcher

Design Document

| Author | Keith Doxey |
| --- | --- |
| Signature |  |
| Date | 22/07/99 |

Master if signed in red

1. Job Information

| DIYHA Project | KAT5 AV Link Transceivers |
| --- | --- |
| Owner | Keith Doxey |
| Customer | For retail sale |

2. Document Information

History

| Author | Issue | Details | Date |
| --- | --- | --- | --- |
| Keith Doxey | 1 | First Draft Issue | 22/07/99 |

Contents

1. PROJECT INFORMATION

2. DOCUMENT INFORMATION

3. DESIGN DECISIONS

4. DESIGN

5. REQUIREMENTS

6. EFFECTS ON OTHER SYSTEMS

7. OTHER TOOLS

3. Design Decisions

The KAT5 AV Link Transceivers will be a range of versatile units to enable the point to point interconnection and distribution of all types of Audio, Video and Computer Video signals around a building equipped with a structured wiring system based on Category 5 Cable.

The units will be available in three different form factors:

Self contained “Black Box” design for “Plug ‘n’ Play”

“Faceplate” design for permanent installation in a wall box

“Bare Board” Uncased printed circuit board for installation into OEM equipment and other equipment housings.

The “Black Box” and “Faceplate” versions will require no specialist knowledge on the part of the end user other than that normally required for interconnecting domestic Audio-visual and Computer equipment.

To use the “Bare Board” version, the user will need knowledge of safe electrical practices and, depending upon the application, will need to know how to interconnect the board to their own equipment.

The KAT5 AV transceivers can be used in conjunction with the KAT5 AV Switchers to enable a fully integrated multi-source multi-zone AV distribution system to be built to your exact requirements.

4. Design

The KAT5 AV Link Transceivers enable the interconnection and distribution of all types of Audio, Video and Computer Video signals around a building equipped with a structured wiring system based on Category 5 Cable. To differentiate between the types of unit there will be a different colour of Power LED to identify the unit

KAT5	AVR   ( Red Power LED)  PSU Required at each end

Stereo Audio and Composite Video with built in Infra Red Repeater

This version sends Left Audio, Right Audio, Composite Video and Infra Red Data over a single CAT5 cable. The units are self-contained boxes that are connected to the relevant source and destination equipment e.g. Video Recorder and Television set.

Connection will be separate RCA phono connectors for Left Right & Video signals. Output connector for the Infra Red Mini-Emitter will be a 3.5mm jack socket.

The audio and video signals from the video recorder are fed into the KAT5 AVR-TX which sends them to the KAT5 AVR-RX connected to the SCART socket or AV inputs of the television set. An Infra Red Receiver built into the AVR-RX relays commands from the viewing room back to the AVR-TX which uses a miniature IR emitter to send the command to the video recorder.

KAT5	AVS   ( Yellow Power LED)  PSU Required at each end

Stereo Audio and S-Video (Y-C) (Separate Luminance and Chrominance signals)

This version sends Left Audio, Right Audio, Video Luminance and Video Chrominance over a single CAT5 cable. Similar in design to the KAT5 AVR but without Infrared capabilities due to the extra video signal. Connection is via RCA Phono sockets for Audio and Phono sockets or 4 pin Mini Din for S-Video. This unit can also be used for 2 mono video sources.

KAT5	CCTV   (Orange Power LED)

PSU required at one end only if not powered from a central video switcher.

Audio, Video, Power and Control signal

This version sends mono audio and video over a single CAT5 cable. Power for the units can be fed from either end and is capable of supplying power to small PCB type security cameras requiring less than 250mA. Additionally there is provision for a control signal which can be used for a PIR Motion detector, a Doorbell press button or as a signal to operate an electrically controlled door.  The PCB for the transmitter is also suitable for mounting in a standard electrical accessory box for wall mounting behind a suitable blank plate. Connection to CCTV-TX is hardwired for tamper proofing and connection to CCTV-RX is by means of RCA Phono Connectors

KAT5	VC  (Orange Power LED) PSU Required at each end.

This version is used for bi-directional Composite Video and Mono Audio.

This version sends mono audio and video over a single CAT5 cable in both directions. Connection to VC-TX and VC-RX is by means of RCA Phono Connectors. Typical applications include Video Conferencing and door entry systems.

KAT5	VGA   ( Green Power LED) PSU Required at each end

Computer Video (RGBS)

This version sends computer video over a single CAT5 cable. The KAT5 VGA-TX requires separate Red, Green and Blue Video signals and either Composite Sync (RGBS) or separate Horizontal and Vertical syncs (RGBHV).

Input is via a 15HD VGA connector (female). Available with a loop though socket to allow the normal monitor to be connected or additional units to be daisy chained. If a unit with loop through is used without a monitor connected the monitor socket must be terminated with a dummy plug.

The KAT5 VGA-RX outputs both composite and separate sync enabling most computer displays and projectors to be driven.

Output is via a 15HD VGA connector (Female). Available with dual output sockets for driving 2 monitors simultaneously.

KAT5	RGB   ( Blue Power LED) PSU Required at each end

RGB (RGBS) or Component (Y-Cr-Cb) Video

This version sends RGB or component video over a single CAT5 cable. The KAT5 RGB-TX requires separate Red, Green and Blue Video signals and Composite Sync (RGBS) or Component Video (Y-Cr-Cb) as output by the latest DVD Players. Input is via four RCA Phono Connectors. The KAT5 RGB uses the same printed circuit board as the KAT5 AVS but is fitted with components for 4 video circuits and has no provision for audio. The 4 pin Mini Din is not fitted.

The KAT5 RGB-RX output is via four RCA Phono Connectors

KAT5	MCA   ( White Power LED)

Multi-Channel Audio

This version sends 4 audio signals over a single CAT5 cable. The KAT5 MCA-TX can be used for four individual mono signals, two stereo signals (LR) or Pro-Logic (LCRS). Input is via four RCA Phono Connectors.

The KAT5 MCA uses the same printed circuit board as the KAT5 AVS but is fitted with components for 4 audio circuits and has no provision for video. The 4 pin Mini Din is not fitted.

The KAT5 MCA-RX output is via four RCA Phono Connectors

KAT5	AV Multi-zone Switcher

The KAT5 AV Switcher is of modular construction.

There will be two types of interface cards.

Input Cards

These will be available in the same variants as the KAT5 AV Transceivers and will receive their input signal via CAT5 cable. There will also be a stripped down version of each input card to receive the input signal in its native format e.g. Composite Video and Stereo Audio. This will allow a piece of equipment such as a satellite receiver located centrally with the switcher to be connected without the additional expense of a KAT5 transceiver.

Output Cards

These will be available in the same variants as the KAT5 AV Transceivers and will send their output signal via CAT5 cable. There will also be a stripped down version of each output card to send the output signal in its native format e.g. Composite Video and Stereo Audio. This will allow a piece of equipment such as an Video Recorder located centrally with the switcher to be connected without the additional expense of a KAT5 transceiver. Some Audio capable versions of the Output cards will have the option of an onboard power amplifier to drive loudspeakers installed throughout the house for Whole House Audio.

Cable Pair Usage

| KAT5 Version | Pair 1 | Pair 2 | Pair 3 | Pair 4 |
| --- | --- | --- | --- | --- |

|  | WH/Bl (Pin 5) | BL/Wh (Pin 4) | WH/Or (Pin 1) | OR/Wh (Pin 2) | WH/Gr (Pin 3) | GR/Wh (Pin 6) | WH/Bn (Pin 7) | BN/Wh (Pin 8) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AVR | Audio Left - | Audio Left + | Video - | Video + | Audio Right - | Audio Right + | Data IR- | Data IR+ |
| AVS Dual Mode | Audio Left – Audio Ch 1 - | Audio Left + Audio Ch 1+ | Video Y- Video Ch 1- | Video Y+ Video Ch 1+ | Audio Right – Audio Ch 2- | Audio Right + Audio Ch 2+ | Video C- Video Ch 2- | Video C+ Video Ch 2+ |
| CCTV | Audio - | Audio + | Video - | Video + | Ground | Control | Power –12V | Power +12V |
| VC | Audio Send - | Audio Send + | Video Send - | Video Send + | Audio Rec - | Audio Rec + | Video Rec - | Video Rec + |
| VGA | Video Blue - | Video Blue + | Video Red - | Video Red + | Video Green - | Video Green+ | Comp Sync - | Comp Sync + |
| RGB (DVD) | Video Blue – (Cb-) | Video Blue + (Cb+) | Video Red – (Cr-) | Video Red + (Cr+) | Video Green- (Y-) | Video Green+ (Y+) | Comp Sync – (Spare) | Comp Sync + (Spare) |
| MCA | Audio Left - | Audio Left + | Audio Centre- | Audio Centre+ | Audio Right - | Audio Right + | Audio Rear - | Audio Rear + |

5. Requirements

This section will describe the hardware, software and networking that will be used.

Hardware:	KAT5 AV Link Transceivers & AV Switcher

Software:	Not Applicable

Network:	Category 5 Cable for interconnection (cable only)

Other:		Some units require a Regulated 12V DC Power supply

6. Effects On Other Systems

There should be no effect on other systems if installed and operated in accordance with the supplied instructions.

Whilst as much care as possible has been taken with these designs, due to the complex and diverse nature of the equipment connected by the user it is impossible to guarantee that all equipment will operate correctly in every case.

7. Other Tools

None.

END OF DOCUMENT